SIYI ZR10 Camera

Jetson Orin Complete Setup & Integration Guide

Ethernet • H.265 RTSP • ROS2 Humble • Qt GUI • Gimbal Control

Version 2.0  |  Includes all known issues and fixes

# System Architecture

The complete system connects a SIYI ZR10 gimbal camera to a Jetson Orin via direct Ethernet, publishes video and gimbal data as ROS2 topics, and streams over WiFi to a Qt-based ground control GUI on a laptop.

ZR10 Camera ──[Ethernet RJ45]──> Jetson Orin ──[WiFi]──> Laptop Qt GUI

IP: 192.168.144.25              IP: 192.168.144.30     ROS_DOMAIN_ID=0

RTSP: :8554/main.264            ROS2 Topics:

Codec: H.265 (NOT H.264)          /zr10/image_raw/compressed

UDP gimbal: :37260                /zr10/gimbal/attitude

/zr10/gimbal/zoom_level

/zr10/stats/cpu_percent

/zr10/stats/cpu_temp

/zr10/stats/wifi_signal

/zr10/stats/wifi_quality

# Prerequisites

## Hardware

- SIYI ZR10 gimbal camera with power cable

- NVIDIA Jetson Orin (any variant)

- SIYI Ethernet-to-RJ45 cable (included in ZR10 box)

- WiFi router/access point on the same network as the laptop

## Software — Jetson Orin

- Ubuntu 22.04 (JetPack 6.x)

- ROS2 Humble

- GStreamer with NVIDIA plugins (included with JetPack)

- Python 3, OpenCV with GStreamer support

- psutil: pip3 install psutil

- pyserial: pip3 install pyserial

## Software — Laptop

- Ubuntu 22.04

- ROS2 Humble

- PyQt5: pip3 install pyqt5

- python3-opencv (apt — NOT pip opencv-python)

- numpy < 2: pip3 install 'numpy<2'

- ros-humble-image-transport-plugins

# Part 1: Hardware Connection & Network Setup

## Step 1: Connect the Camera

Connect the ZR10 Ethernet port directly to the Jetson Orin Ethernet port using the included Ethernet-to-RJ45 cable. No switch or router is needed for the camera connection.

⚠ NOTE: The ZR10 status LED should be solid green when powered on correctly.

## Step 2: Find the Ethernet Interface Name

On Jetson Orin the Ethernet interface is NOT named eth0. Run:

ip a

Look for an interface starting with en (e.g., eno1, enp3s0). On Jetson Orin it is typically eno1.

⚠ NOTE: Common mistake: using eth0 which does not exist. Always run 'ip a' first to confirm the interface name.

## Step 3: Assign Static IP Address

Assign a static IP on the same subnet as the ZR10 (192.168.144.x):

sudo ip addr add 192.168.144.30/24 dev eno1

# Verify:

ip addr show eno1

⚠ NOTE: This assignment is temporary and resets on reboot. See Step 4 for the permanent fix.

## Step 4: Make IP Persistent (Permanent Fix)

sudo nmcli con add type ethernet ifname eno1 con-name zr10-cam ip4 192.168.144.30/24

sudo nmcli con up zr10-cam

# Verify permanent connection:

nmcli con show zr10-cam

✓ TIP: Check that connection.autoconnect is 'yes' in the output — this ensures the IP is assigned after every reboot.

## Step 5: Verify Camera is Reachable

Always ping through the specific interface to avoid routing through WiFi:

ping -I eno1 192.168.144.25

Expected: replies with ~0.4ms latency and 0% packet loss.

## Known Issue: Ping Routes Through Internet

Symptom: ping replies come from an external IP (e.g. 66.97.22.4) instead of the camera.

Cause: The IP was not assigned to eno1, so the system routes through the WiFi default gateway.

Fix:

# Check IP is assigned

ip addr show eno1

# If 192.168.144.30 is missing, re-add it:

sudo ip addr add 192.168.144.30/24 dev eno1

# Force ping through eno1:

ping -I eno1 192.168.144.25

## ZR10 Default Network Settings

| Camera IP | 192.168.144.25 |
| --- | --- |
| RTSP Port | 8554 |
| Stream URL | rtsp://192.168.144.25:8554/main.264 |
| Actual Codec | H.265 (HEVC) — despite .264 in the URL name |
| Gimbal UDP Port | 37260 |
| Jetson IP | 192.168.144.30 (assigned) |

# Part 2: RTSP Video Stream

## Step 6: Test Stream with GStreamer

Before setting up ROS2, verify the stream works directly:

gst-launch-1.0 rtspsrc location=rtsp://192.168.144.25:8554/main.264 latency=0 \

! rtph265depay ! h265parse \

! nvv4l2decoder ! nvvidconv \

! video/x-raw,format=BGRx ! videoconvert \

! autovideosink

A video window should appear with the live feed.

## Known Issue: rtph264depay Linking Failed

Symptom: 'failed delayed linking some pad of GstRTSPSrc' error.

Cause: The URL says main.264 but the camera actually streams H.265. Using rtph264depay causes a mismatch.

Fix: Always use rtph265depay and h265parse regardless of the URL name.

# WRONG:

! rtph264depay ! h264parse

# CORRECT:

! rtph265depay ! h265parse

⚠ NOTE: The .264 in the URL is just SIYI's naming convention — the actual codec is always H.265.

## Known Issue: Verbose Caps Output

To check what codec the camera is actually sending:

gst-launch-1.0 -v rtspsrc location=rtsp://192.168.144.25:8554/main.264 latency=0 ! fakesink 2>&1 | head -30

Look for encoding-name=(string)H265 in the output to confirm the codec.

# Part 3: ROS2 Package Setup

## Step 7: Create the Package

cd ~/ros2_ws/src

ros2 pkg create zr10_camera \

--build-type ament_python \

--dependencies rclpy sensor_msgs cv_bridge image_transport

## Step 8: Package Structure

~/ros2_ws/src/zr10_camera/

├── launch/

│   ├── zr10_stream.launch.py    ← main launch (video + gimbal + stats)

│   └── zr10_raw.launch.py       ← optional raw image launch

├── zr10_camera/

│   ├── __init__.py

│   ├── zr10_publisher.py        ← compressed video publisher

│   ├── zr10_gimbal.py           ← gimbal UDP control node

│   ├── zr10_raw_publisher.py    ← optional raw image node

│   └── zr10_system_stats.py     ← Jetson system stats node

├── package.xml

├── setup.cfg

└── setup.py

## Step 9: setup.py Entry Points

Make sure all nodes are registered in setup.py:

entry_points={

'console_scripts': [

'zr10_publisher     = zr10_camera.zr10_publisher:main',

'zr10_gimbal        = zr10_camera.zr10_gimbal:main',

'zr10_raw_publisher = zr10_camera.zr10_raw_publisher:main',

'zr10_system_stats  = zr10_camera.zr10_system_stats:main',

],

},

## Step 10: Build

cd ~/ros2_ws

colcon build --packages-select zr10_camera

source install/setup.bash

# Part 4: Video Publisher Node

## Pipeline Design

The publisher uses hardware-accelerated H.265 decoding via nvv4l2decoder, scales to 480p to reduce CPU load, and encodes JPEG directly with OpenCV. Raw image publishing is disabled to save CPU.

# GStreamer pipeline (480p, hardware decode, no videoconvert):

rtspsrc location={url} latency=0

! rtph265depay ! h265parse

! nvv4l2decoder ! nvvidconv

! video/x-raw, width=854, height=480, format=BGRx

! appsink drop=1 max-buffers=1

# In Python: drop X channel with numpy slice (no videoconvert needed)

bgr = frame[:, :, :3]

## Known Issue: High CPU Usage (80-100%)

Symptom: zr10_publisher shows 80-100% in top even at 15fps.

Root cause: top shows per-core usage. On a 6-core Jetson Orin, 80% in top = ~13% system average. The Jetson is not actually overloaded.

| top shows | Per-core % (80% = one core busy) |
| --- | --- |
| psutil reports | System average % (80% ÷ 6 cores = ~13%) |
| Real system load | ~13% average — healthy |
| Resolution to reduce CPU | Use 480p (854x480) instead of 1080p |
| Frame rate to reduce CPU | 15fps is a good balance for streaming |

⚠ NOTE: nvjpegenc (hardware JPEG encoding) is not available on all JetPack versions. If the pipeline fails, fall back to cv2.imencode with numpy BGRx slicing.

## Known Issue: Multiple Publisher Instances

Symptom: Video freezes or glitches. ros2 topic info shows Publisher count > 1.

Cause: Pressing Ctrl+C only stops the launch session but leaves child processes running. Each relaunch adds another publisher.

Fix: Always kill existing instances before relaunching:

pkill -f zr10_publisher

pkill -f zr10_gimbal

pkill -f zr10_system_stats

# Verify clean state:

ros2 topic info /zr10/image_raw/compressed

# Should show: Publisher count: 0

# Part 5: Gimbal UDP Control

## Protocol Overview

The ZR10 accepts SIYI SDK commands via UDP on port 37260. No UART or serial connection is needed — the same Ethernet cable used for video also carries gimbal commands.

| Camera IP | 192.168.144.25 |
| --- | --- |
| UDP Port | 37260 |
| Protocol | SIYI SDK (custom binary) |
| Baud rate | N/A (UDP over Ethernet) |
| Yaw range | -135° to +135° |
| Pitch range | -90° to +25° |
| Zoom range | 1.0x to 30.0x |

## Key Commands

| CMD_ID | Function | Data |
| --- | --- | --- |
| 0x07 | Speed rotation | int8 yaw_speed, int8 pitch_speed (-100 to 100) |
| 0x0E | Absolute angle | int16 yaw×10, int16 pitch×10 |
| 0x08 | Center gimbal | 0x01 |
| 0x05 | Manual zoom | 1=in, -1=out, 0=stop |
| 0x0F | Absolute zoom | uint8 integer, uint8 decimal |
| 0x0D | Request attitude | no data |
| 0x04 | Auto focus | 0x01 + 4 zeros |

## ROS2 Topics

| Topic | Type | Direction | Description |
| --- | --- | --- | --- |
| /zr10/gimbal/center | std_msgs/Bool | Publish | True = center gimbal |
| /zr10/gimbal/angle | geometry_msgs/Vector3 | Publish | x=yaw, y=pitch (degrees) |
| /zr10/gimbal/rotate | geometry_msgs/Vector3 | Publish | x=yaw_speed, y=pitch_speed |
| /zr10/gimbal/zoom | std_msgs/Float32 | Publish | Absolute zoom 1.0-30.0 |
| /zr10/gimbal/zoom_step | std_msgs/Int8 | Publish | 1=in, -1=out, 0=stop |
| /zr10/gimbal/attitude | geometry_msgs/Vector3 | Subscribe | x=yaw, y=pitch, z=roll |
| /zr10/gimbal/zoom_level | std_msgs/Float32 | Subscribe | Current zoom level |

## Test Commands (from laptop)

# Center gimbal

ros2 topic pub --once /zr10/gimbal/center std_msgs/msg/Bool "data: true"

# Set absolute angle (yaw=0, pitch=-45°)

ros2 topic pub --once /zr10/gimbal/angle geometry_msgs/msg/Vector3 "{x: 0.0, y: -45.0, z: 0.0}"

# Zoom to 5x

ros2 topic pub --once /zr10/gimbal/zoom std_msgs/msg/Float32 "data: 5.0"

# Monitor attitude feedback

ros2 topic echo /zr10/gimbal/attitude

# Part 6: System Stats Node

## Published Topics

| Topic | Unit | Description |
| --- | --- | --- |
| /zr10/stats/cpu_percent | % | System-wide CPU average across all cores |
| /zr10/stats/cpu_total | % | Total CPU summed across cores (matches top) |
| /zr10/stats/mem_percent | % | Memory usage |
| /zr10/stats/cpu_temp | °C | CPU temperature from thermal_zone0 |
| /zr10/stats/wifi_signal | dBm | WiFi signal strength |
| /zr10/stats/wifi_quality | % | WiFi link quality |
| /zr10/stats/wifi_bandwidth | Mbps | WiFi bandwidth (rolling average) |
| /zr10/stats/eth_bandwidth | Mbps | Ethernet bandwidth (rolling average) |
| /zr10/stats/diagnostics | - | DiagnosticArray with all stats |

## Known Issue: CPU Temperature Returns -1.0

Symptom: /zr10/stats/cpu_temp always shows -1.0.

Cause: The default thermal zone path differs on Jetson Orin.

Fix: Use thermal_zone0 which maps to cpu-thermal on Jetson Orin:

# Verify thermal zones:

paste <(cat /sys/class/thermal/thermal_zone*/type) \

<(cat /sys/class/thermal/thermal_zone*/temp)

# Expected output:

# cpu-thermal   48437   ← zone 0, divide by 1000 = 48.4°C

# gpu-thermal   48625

# In zr10_system_stats.py use:

with open('/sys/class/thermal/thermal_zone0/temp', 'r') as f:

return float(f.read().strip()) / 1000.0

## Understanding CPU % Readings

The cpu_percent topic and top report different numbers — both are correct but measure different things:

| top (per-process) | Shows % of ONE core. 60% = one core is 60% busy. |
| --- | --- |
| psutil cpu_percent | System average across ALL cores. 60% on one core ÷ 6 = 10% average. |
| psutil cpu_total | Sum across all cores. Matches top's overall display. |

At 480p 15fps the zr10_publisher typically uses ~60% of one core (~10% system average). The Jetson is healthy with plenty of headroom for detection algorithms.

# Part 7: Image Transport Plugins

## Installation

Required on both Jetson and laptop for compressed image transport:

sudo apt install ros-humble-image-transport-plugins

## Known Issue: apt 404 Error on Jetson (arm64)

Symptom: 404 Not Found when installing ros-humble-image-transport-plugins on Jetson.

Cause: The arm64 package is not always available in the ROS apt mirror.

Fix: Build from source:

cd ~/ros2_ws/src

git clone https://github.com/ros-perception/image_transport_plugins.git -b humble

cd ~/ros2_ws

colcon build --packages-select compressed_image_transport compressed_depth_image_transport

source install/setup.bash

## Known Issue: rqt Crashes on Compressed Topic

Symptom: rqt_image_view crashes with 'invalid allocator' when selecting /zr10/image_raw/compressed.

Cause: rqt remembers the last selected topic and tries to auto-subscribe on startup before the compressed plugin loads.

Fix 1: Clear rqt config:

rm -rf ~/.config/ros.org/

Fix 2: Launch rqt forcing raw transport:

ros2 run rqt_image_view rqt_image_view --ros-args -p image_transport:=raw

Then manually select /zr10/image_raw from the dropdown.

# Part 8: Qt Ground Control GUI

## Installation — Laptop

# Install PyQt5

pip3 install pyqt5

# Install system OpenCV (NOT pip version — causes Qt conflict)

pip3 uninstall opencv-python opencv-python-headless

sudo apt install python3-opencv

# Fix NumPy version conflict

pip3 install 'numpy<2'

# Install Qt Designer

sudo apt install qttools5-dev-tools

## Known Issue: Qt Platform Plugin xcb Crash

Symptom: 'Could not load Qt platform plugin xcb' when running the GUI.

Cause: pip opencv-python bundles its own Qt which conflicts with the system PyQt5.

Fix: Uninstall pip OpenCV and use system OpenCV instead:

pip3 uninstall opencv-python opencv-python-headless

sudo apt install python3-opencv

## Known Issue: NumPy 2.x Incompatibility

Symptom: 'numpy.core.multiarray failed to import' or '_ARRAY_API not found' error.

Cause: system OpenCV on Ubuntu 22.04 was compiled against NumPy 1.x and is incompatible with NumPy 2.x.

Fix:

pip3 install 'numpy<2'

# Verify:

python3 -c "import numpy; print(numpy.__version__)"

# Should show 1.24.x or 1.26.x

⚠ NOTE: This does not affect ROS2 — ROS2 Humble uses its own internal NumPy installation.

## Running the GUI

Both files must be in the same directory:

your_folder/

├── zr10_gui.py

└── zr10_control.ui

cd your_folder

source /opt/ros/humble/setup.bash

export ROS_DOMAIN_ID=0

python3 zr10_gui.py

## GUI Controls Reference

| Control | Type | Function |
| --- | --- | --- |
| z_com_slider | Horizontal slider | Yaw control, real-time (-135° to 135°) |
| y_com_slider | Vertical slider | Pitch control, real-time (-90° bottom to 25° top) |
| zoom_com_dial | Dial | Zoom control, real-time (1.0x to 30.0x) |
| des_x / des_y | Text inputs | Type exact angles in degrees |
| Set Gimbal Angle | Button | Send typed angles + sync sliders |
| Reset Gimbal | Button | Center gimbal + reset sliders to 0 |
| Set Zoom | Button | Send typed zoom + sync dial |
| Reset Zoom | Button | Zoom to 1x + reset dial |
| gimbal_control_status | Status label | Green=Normal, Blue=Set, Red=Lost Connection |
| video_label | Video display | Live feed or red No Signal when disconnected |

# Part 9: WiFi Streaming

## Setup ROS Domain ID

Both machines must use the same domain ID:

# On both Jetson and laptop:

export ROS_DOMAIN_ID=0

# Make permanent — add to ~/.bashrc:

echo 'export ROS_DOMAIN_ID=0' >> ~/.bashrc

echo 'source /opt/ros/humble/setup.bash' >> ~/.bashrc

echo 'source ~/ros2_ws/install/setup.bash' >> ~/.bashrc

source ~/.bashrc

## Bandwidth Comparison

| Topic | Format | ~Bandwidth | Recommended for WiFi |
| --- | --- | --- | --- |
| /zr10/image_raw | Raw BGR8 | ~500 Mbps | No — too large |
| /zr10/image_raw/compressed | JPEG compressed | ~5-20 Mbps | Yes |

⚠ NOTE: Always use /zr10/image_raw/compressed over WiFi. Raw image topic will freeze within seconds.

## Known Issue: Video Glitches Over WiFi

Possible causes and fixes:

| Multiple publisher instances | pkill -f zr10_publisher, then relaunch once |
| --- | --- |
| Chrome/browser using CPU on laptop | Close browser tabs during streaming |
| Resolution too high | Use 480p (854x480) — reduces bandwidth by 56% |
| Frame rate too high | Use 15fps — sufficient for monitoring |
| WiFi congestion | Check iwconfig for signal quality — should be above 60% |

# Part 10: Launch Files & Running

## Main Launch (always run first)

ros2 launch zr10_camera zr10_stream.launch.py

# With custom options:

ros2 launch zr10_camera zr10_stream.launch.py fps:=20.0 jpeg_quality:=90

This starts: zr10_publisher + zr10_gimbal + zr10_system_stats

## Optional Raw Image Launch

# In a separate terminal (only when needed for detection):

ros2 launch zr10_camera zr10_raw.launch.py

# With custom fps:

ros2 launch zr10_camera zr10_raw.launch.py fps:=3.0

## Verify Everything is Running

# Check all topics are publishing:

ros2 topic list

# Check frame rate:

ros2 topic hz /zr10/image_raw/compressed

# Expected: ~15 Hz

# Check gimbal attitude:

ros2 topic echo /zr10/gimbal/attitude

# Check system stats:

ros2 topic echo /zr10/stats/cpu_temp

# Part 11: ROS2 Bag Recording

## Record

# Record compressed video only:

ros2 bag record /zr10/image_raw/compressed -o zr10_bag

# Record all relevant topics:

ros2 bag record \

/zr10/image_raw/compressed \

/zr10/gimbal/attitude \

/zr10/gimbal/zoom_level \

/zr10/stats/cpu_percent \

/zr10/stats/cpu_temp \

/zr10/stats/wifi_signal \

-o zr10_bag

# Record with duration limit (30 seconds):

ros2 bag record /zr10/image_raw/compressed -o zr10_bag --duration 30

## Replay

# Normal replay:

ros2 bag play zr10_bag

# Half speed:

ros2 bag play zr10_bag --rate 0.5

# Loop:

ros2 bag play zr10_bag --loop

# Inspect bag info:

ros2 bag info zr10_bag

✓ TIP: Run the Qt GUI during bag replay — all topics feed into it exactly as if the Jetson is live. Great for post-mission review.

# Part 12: GitHub Repository Setup

## Jetson Package .gitignore

__pycache__/

*.py[cod]

*.egg-info/

/build/

/install/

/log/

*.bag

*.db3

rosbag2_*/

*_recording/

.vscode/

.DS_Store

*.log

## Qt GUI .gitignore

__pycache__/

*.py[cod]

*.egg-info/

.venv/

venv/

*.user

.vscode/

.DS_Store

*.log

*.db3

*.bag

rosbag2_*/

## Push to GitHub

# Jetson repo:

cd ~/ros2_ws/src/zr10_camera

git init

git add .

git commit -m "Initial commit: ZR10 ROS2 camera and gimbal control"

git remote add origin https://github.com/yourusername/zr10_camera.git

git push -u origin main

# Laptop GUI repo:

cd your_gui_folder

git init

git add .

git commit -m "Initial commit: ZR10 Qt GUI ground control"

git remote add origin https://github.com/yourusername/zr10_ground_control_gui.git

git push -u origin main

# Complete Troubleshooting Reference

| Problem | Cause | Fix |
| --- | --- | --- |
| Cannot find device eth0 | Interface named eno1 on Jetson Orin | Run ip a to find correct name |
| Ping routes through internet (66.97.x.x) | IP not assigned to eno1 | sudo ip addr add 192.168.144.30/24 dev eno1, then ping -I eno1 |
| RTSP timeout — Could not open resource | IP reset after reboot | Use nmcli to make IP permanent (see Step 4) |
| rtph264depay linking failed | Camera streams H.265 despite .264 URL | Use rtph265depay and h265parse |
| source ~/ros2_ws/install/setup.bash fails | Wrong path (missing ~) or not built yet | Use ~/ros2_ws not /ros2_ws, build first with colcon |
| rqt crashes on compressed topic | image_transport_plugins missing or rqt config stale | rm -rf ~/.config/ros.org/ then relaunch rqt |
| apt 404 on arm64 for image_transport_plugins | Package not available for arm64 in mirror | Build from source (see Part 7) |
| Qt xcb plugin crash | pip opencv-python bundles conflicting Qt | pip3 uninstall opencv-python, sudo apt install python3-opencv |
| numpy.core.multiarray failed to import | NumPy 2.x incompatible with system OpenCV | pip3 install 'numpy<2' |
| CPU temp shows -1.0 | Wrong thermal zone path | Use /sys/class/thermal/thermal_zone0/temp (cpu-thermal on Orin) |
| Video freezes, Publisher count > 1 | Old instances still running after Ctrl+C | pkill -f zr10_publisher before relaunching |
| zr10_gimbal executable not found | Entry point missing from setup.py | Add zr10_gimbal = zr10_camera.zr10_gimbal:main to setup.py |
| nvjpegenc pipeline fails | Not available on all JetPack versions | Use cv2.imencode with numpy BGRx slice instead |
| Gimbal status shows Lost Connection | zr10_gimbal node not running or UDP unreachable | Check node is launched, ping 192.168.144.25:37260 |

# Quick Reference — Full Command Cheat Sheet

# ── Network ──────────────────────────────────────────────────────────

ip a                                          # find interface name

sudo ip addr add 192.168.144.30/24 dev eno1  # assign IP (temp)

sudo nmcli con add type ethernet ifname eno1 con-name zr10-cam ip4 192.168.144.30/24

sudo nmcli con up zr10-cam                   # activate permanent IP

ping -I eno1 192.168.144.25                  # verify camera reachable

# ── GStreamer test ───────────────────────────────────────────────────

gst-launch-1.0 rtspsrc location=rtsp://192.168.144.25:8554/main.264 latency=0 \

! rtph265depay ! h265parse ! nvv4l2decoder ! nvvidconv \

! video/x-raw,format=BGRx ! videoconvert ! autovideosink

# ── ROS2 ─────────────────────────────────────────────────────────────

source /opt/ros/humble/setup.bash

source ~/ros2_ws/install/setup.bash

export ROS_DOMAIN_ID=0

# Build

cd ~/ros2_ws && colcon build --packages-select zr10_camera

# Launch (main)

ros2 launch zr10_camera zr10_stream.launch.py

# Launch (raw image — optional)

ros2 launch zr10_camera zr10_raw.launch.py

# Kill all nodes before relaunch

pkill -f zr10_publisher && pkill -f zr10_gimbal && pkill -f zr10_system_stats

# ── Verification ─────────────────────────────────────────────────────

ros2 topic list                               # check all topics

ros2 topic hz /zr10/image_raw/compressed      # should be ~15 Hz

ros2 topic echo /zr10/gimbal/attitude         # check gimbal feedback

ros2 topic echo /zr10/stats/cpu_temp          # check ~48°C

# ── Gimbal commands ──────────────────────────────────────────────────

ros2 topic pub --once /zr10/gimbal/center std_msgs/msg/Bool "data: true"

ros2 topic pub --once /zr10/gimbal/angle geometry_msgs/msg/Vector3 "{x: 0.0, y: -30.0, z: 0.0}"

ros2 topic pub --once /zr10/gimbal/zoom std_msgs/msg/Float32 "data: 5.0"

# ── Bag recording ────────────────────────────────────────────────────

ros2 bag record /zr10/image_raw/compressed /zr10/gimbal/attitude -o zr10_bag

ros2 bag play zr10_bag

ros2 bag info zr10_bag

# ── Qt GUI ───────────────────────────────────────────────────────────

cd your_gui_folder && python3 zr10_gui.py

# Healthy System Benchmarks

| Metric | Expected Value | Status |
| --- | --- | --- |
| Ping latency | ~0.4ms | Excellent |
| RTSP stream | H.265, port 8554 | Normal |
| Video topic rate | ~15 Hz | Normal |
| CPU (system avg) | ~10-15% | Healthy |
| CPU (one core) | ~60-70% | Normal for publisher |
| CPU temperature | ~48°C under load | Normal |
| Memory usage | ~30-35% | Healthy |
| WiFi signal | > -60 dBm | Good |
| WiFi quality | > 60% | Good |
| WiFi bandwidth | ~5-20 Mbps | Normal for 480p 15fps |